Leonardo Da Pinchy

Bryan Hong, Aaron Tang, Jonathan Waller

Problem/Scope

Humans have trained animals to create artwork, but making strokes on a canvas is much more difficult for robots with mechanical dexterity that hasn’t undergone millions of years of evolution. Machines that are capable of drawing typically have only 2 to 3 degrees of freedom (DoF), with limited z-axis motion (i.e. CNC machine), and assume the surface is fixed. However, they fail to adapt when drawing surfaces are positioned between drawings, or repositioned, in 3D space. Though many industrial domains demand high DoFs, drawing provides a sandbox that requires equally delicate, precise, and continuous contact with the physical world.

The scope of this project is to build a 6 DoF robotic arm capable of automatically adapting to arbitrarily positioned flat surfaces in space, selecting different colored Sharpies from a tool rack, and drawing simple user-submitted drawings. Scope boundary: The core project focuses on planar surfaces (e.g., clipboards propped up at various angles) with a fixed operating space; deformable or non-planar 3D geometry is outside the primary scope.

Requirements

Our target users are digital artists and educators who sketch artwork or lesson notes on a computer and want an automated tool to recreate them on surfaces like a canvas or whiteboard. Users should be able to draw on a website, have the arm automatically figure out where the drawing board is located without manual setup, and let the arm automatically switch between different marker colors.

The main technical challenges we foresee are accurately locating a tilted surface, maintaining gentle contact with the paper, and repeatedly picking up markers whose tip positions may vary. These challenges would be addressed using camera-based surface detection, a spring based force-sensing marker holder, and strong pincers for holding the marker in place.

Performance Specifications:

Emergency Stop: Stop the robot from breaking itself or the environment

Board’s Allowed Location: Approx. 0.4 m x 0.4 m x 0.4 m

Board’s Allowed Orientation: 0° to 90° tilt from horizontal, with rotation within the board plane

Drawing Space: Normal US printer paper, around 21.6 cm x 27.95 cm

Drawing Format: SVG line drawing, up to two colors, 50 separate strokes, and 100 cm total stroke length

Line Accuracy: Maintain drawing path accuracy within approx. 2.0 mm of desired line

Pick-Up Reliability: Swap success rate of approx. 90% or better

Web Response: Create a queue that lets the robot be reset with new papers, positions, etc. between runs

Surface Contact: Maintain continuous pen pressure against paper without tearing or slipping.

Motor type: Servos for end effector actuation (grabber), possibly steppers or servos for joints

Unknown Requirements:

Contact Force: Exact normal force needed at brush tip (unknown)

Power: Total power supply requirements (unknown)

Thermals: We do not believe thermal issues will be a problem

Implementation
The system will use a multi-joint robotic arm with servo motors and a servo-powered marker gripper. A camera above the board will first estimate the board and markers’ position relative to the arm and operational space. The arm can also be calibrated by touching three points on the board to measure its exact position and angle, since three points define a flat surface. A spring-loaded marker holder and force sensor will help maintain steady contact without damaging the paper. A Raspberry Pi will run the web server, drawing queue, camera processing, and SVG path planning. A microcontroller will control the motors and read the joint and force sensors, and the two boards will communicate through USB. Markers will be stored in two fixed rack positions for changing color. A hardwired emergency stop will directly disable motor power.

User Image (Converted to a 2d map of points)

Scale and transpose 2d points onto the surface in 3d points

Surface Position

Marker Positions

Inverse kinematics + Cartesian Path Interpolation to create the current drawings full path (ROS2 computeCartesianPath)

Find marker grab points

Robot execution

Testing

The system will need to go through several individually tested stages:

- Identifying and selecting the correct color pen

- Calculating coordinates and assigning colors to the image

- Drawing the image

- Returning the pen to its correct position on the tool rack

A series of tasks ordered according to increasing complexity will be given to the system to perform. For example, the system would first need to distinguish between several different-colored Sharpie pens with minimal error. Then, the robotic arm would need to move over the selected pen and pick it out of the tool rack. We would also test the ability of the arm to draw on a flat surface before we begin adjusting the angle of the drawing surface.

MVP

Our MVP will be a custom 6-DoF robotic arm that uses an external camera and AprilTag markers to calibrate to a propped planar surface, picks up a specified Sharpie marker from a rack, and renders a line drawing submitted by a user via a web interface onto paper within its operating space.